Adaptive Multi-Cell Battery Intelligence Engine

Internship Task | ElevanceSkills

Student Name: Poorwaksha Panse

Date: 29 May 2026

- Objective

Implement a production-grade battery intelligence engine capable of monitoring a simulated 4-cell lithium battery pack in real time using Arduino Uno on the Wokwi platform.

2. System Architecture

The system uses 5 modular software components:

- Module 1: ADC reading with 8x oversampling for noise reduction

- Module 2: Voltage calculation using the voltage divider formula

- Module 3: Pack statistics (average, total, imbalance %)

- Module 4: Health classification state machine

- Module 5: Formatted serial monitor output

3. Circuit Design

Each of the 4 cells is simulated by a potentiometer. The output voltage is scaled using a resistor divider (R1 = 100kΩ, R2 = 33kΩ) so a 4.2V cell maps to ≈1.04V, safely within the Arduino's 5V ADC range. All 4 divider outputs connect to pins A0–A3.

4. Health Classification Logic

| State | Condition |
| --- | --- |
| Healthy | Imbalance < 2% |
| Minor Imbalance | 2% ≤ imbalance < 5% |
| Critical Imbalance | imbalance ≥ 5% |
| Pack Failure | Any cell < 3.0V or > 4.3V |

5. Results

The system successfully identifies the weakest and strongest cells, calculates State of Charge (SoC) per cell, and prints a complete diagnostic report every 2 seconds over the serial monitor.

6. Live Simulation Link [ADAPTIVE-MULTI-CELL-BATTERY-INTELLIGENCE-ENGINE Copy - Wokwi ESP32, STM32, Arduino Simulator]

7. GitHub Repository [Poorwaksha/ADAPTIVE-MULTI-CELL-BATTERY-INTELLIGENCE-ENGINE: Production-grade 4-cell Lithium-ion Battery Management System (BMS) simulation using Arduino UNO in Wokwi with real-time voltage monitoring, battery health analysis, imbalance detection, LCD dashboard, LED alerts, buzzer warning system, and modular embedded architecture.]